Feasibility study on the use of recycled materials for prototyping purposes: a comparative study based on the tensile strength

Victor M. López

Diego Carou

Fabio A. Cruz S.

3D printing is seen as a disruptive technology and continues to expand the design space boundaries for prototypes and final products. Sustainability is one of the major objectives for manufacturing and the use of recycled materials is becoming a relevant strategy, particularly for improving material resource efficiency. This paper attempts to evaluate the suitability of the substitution of virgin polylactic acid (PLA) for recycled PLA. An experimental plan divided into three phases to evaluate the tensile strength of the specimens was described. The results showed that recycled PLA may be used thanks to the similar resistance, even though this is slightly lower than the virgin material. In addition, the infill density and the orientation parameters played a major role on the response. A retention of 58.1 % of the resistance using an infill density of 40 % was evidenced. In addition, because of the anisotropy, it was found that the horizontal orientation allowed to attain a higher maximum load, while the vertical orientation provided a lower value These are relevant insights for prescriptions of the 3D printing parameters guaranteeing minimum quality conditions in prototyping.

Keywords: 3D printing; Prototyping; Distributed recycling; Tensile strength; PLA

# 1	Introduction

Fused filament fabrication (FFF) is a major additive manufacturing technology, which has found considerable number of applications in different types of manufacturing sectors.1,2 The layer-by-layer principle of manufacturing objects enables a higher degree of flexibility in the product design phase.3 The set of several available printing technologies4 is pushing forward advantages such as the mass customization5 with complex geometries that involve a great deal of detail, a combination of different materials,6 a reduction in the need for assembly and a high utilization rate of raw materials.7

Nowadays, there is a need to find ways to reduce the ecological impact of manufacturing processes, pursuing sustainable and clean manufacturing processes.8,9 Researchers are making efforts to identify opportunities for 3D printing on the circular economy paradigm.10 Moreover, due to the fact that plastic is one of the most highly used materials in the 3D printing industry11 and given its non-biodegradable nature, plastic is one the most abundant types of waste produced. The environmental impact of plastics is well documented in the main literature.12 Thus, reducing the consumption of plastics is of great importance.

A major body of literature arising from the fields of engineering, human–computer interaction, design thinking and software development13 validates the rationale for the prototyping phase in the early design phases of product development. According to the prototyping theory, different kinds of prototypes are needed during the new product development phases (e.g. prototypes for desirability, for feasibility, and for viability)14 with the purpose of reducing uncertainties, exploring new ideas, increasing feasibility and/or engaging with users.15 On that basis, a prototype is achieved in terms of certain modelling aims: Model to Link, Model to Test, Model to Communicate, Model to Decide and Model to Interact.14 The use of digital tools allows designers to create highly flexible prototypes that facilitate short learning cycles at an affordable cost. Moreover, the use of 3D printing technology enables the materialization aspect. Regardless of whether the printed object is functional or not, it is found to be valuable in design decisions.13 However, there is a gap in the literature in terms of sustainable manufacturing using 3D printing in the early design phases.9 Although the technology offers high efficiency in the use of materials, the democratization of this technology could cause a rebound impact due to the increasing generation and disposal of huge amounts of waste or polluting emissions to fabricate the virgin feedstock required, particularly, in prototyping. Without a doubt, the roots of FFF are linked to the rapid prototyping concept16 and in recent years it has been widely adopted to create functional objects for their designs. Therefore, one question that remains is how to define the most favorable printing conditions to create prototypes in the early phases without compromising the mechanical properties, even for recycled feedstocks.

Studies on the technical viability of recycled materials as substitutes for conventional virgin materials are still limited to particular applications.17,18 It is important to note that, in most cases, prototypes do not require excellent mechanical properties but the minimum to be handled to allow inspection and measurement. Thus, the type of material used and its amount can be further optimized when it comes to prototyping. The mechanical properties are critical for engineering parts, particularly, for 3D printed parts because of the anisotropy,19 which can influence the ultimate tensile strength (UTS) up to about 47 % as it pertains to the manufacturing parameters.20 Using a systematic literature review, Popescu et al.21 identified key parameters that influence the printed parts, including the raster-to-raster air gap, raster angle, layer thickness, infill density and build orientation.

In general terms, it is found that for low values of layer height, the tensile strength of the material is improved.22,23 Similarly, Yao et al.24 identified the importance of the printing orientation in the UTS. Thus, the alignment of the tensile load with the longitudinal axis of the printed fiber will maximize the UTS. According to Alafaghani et al.25 a higher extrusion temperature, an optimized layer thickness, a triangular filling pattern and a higher filling level maximize the strength of the parts. Regarding the printing speed, it has been determined that a higher printing speed with a higher layer thickness leads to lower part strength.

In the literature, distributed recycling via additive manufacturing (DRAM) approach emphasizes the technical steps required to reuse plastic waste through the recycling chains for material-extrusion-based 3D printing.17,26 The use of recycled material, either in the form of raw material or blended with virgin material, is a method of special interest to contribute to sustainable manufacturing.27 In the DRAM methodology, consumers have an economic incentive to recycle. This is because they can use their waste as feedstock for a wide range of consumer products that can be produced for a fraction of the conventional cost of the equivalent products. Moreover, 3D printing is especially well suited because it enables the production of parts with (almost) no waste and could reduce the waste related to the material by more than 40 %, reusing 95 % of the unused material.28 Currently, most of the cost of 3D printing is associated with filament.29 By recycling raw materials such as polylactic acid (PLA), one of the most frequently used materials in 3D printing, it is possible to reduce the carbon dioxide emissions that are incurred by transport to landfills or shipping to customers, offering environmental benefits.30

It is important to evaluate the properties of the recycled materials before substituting virgin for recycled materials. The use of recycled materials is still uncertain because of the potential changes in the material properties when recycling.31 Several authors have studied the printing cycles that PLA can withstand until it loses much of its properties.27,32–34 There is an agreement that PLA adequately withstands two printing cycles since after a third cycle or more the mechanical properties and viscosity decreased considerably. The increase in crystallinity and melting enthalpy and the decrease in cold crystallization enthalpy are attributed to the 3D printing process. For instance, Kumar et al.35 compared the elongation at break, load at break, flow index, Young’s modulus and breaking stress of recycled acrylonitrile butadiene styrene (ABS), high impact polystyrene (HIPS) and PLA. The PLA showed the highest elongation at break along with the ABS. In addition, the PLA had a higher breaking load and breaking stress, although a smaller Young’s modulus. Likewise, Babagowda et al.36 studied the influence of the percentage of recycled PLA used in the filament, from 10 to 50 %, showing that the smaller the percentage the higher the ultimate tensile strength. In summary, the recycling of PLA has certain limitations due to the reduction in the molecular weight with its reuse, resulting in degradation and a decrease in mechanical properties.37 The viscosity is also reduced with each printing cycle, but it could be corrected by adding virgin plastic.27,38

It might be uncertain whether a set of optimal parameters for a machine/material/application combination can be transferred to other 3D printers due to the issue of intra-3D printer variability and the variations of the quality of the recycled material. Robust methods are needed to develop standards to validate the process setting minimal requirements for the resistance, dimensional accuracy, replicability and minimum feature size among the 3D printing technologies.39 Besides, considering the open-source nature of FFF technology, standardized experimental protocols are relevant to enable benchmarking and to serve as a guide for machine selection.40,41 Therefore, it is crucial to identify the most important parameters that may affect the process quality.42 The present study proposes a methodology in three phases to evaluate the tensile strength of both conventional and recycled PLA materials. The objective is the assessment of the suitability of the recycled PLA as a replacement in prototyping, though its use may be further extended to other applications. To do so, this research is based on a comprehensive experimental study with three main phases to evaluate the influence of several printing parameters on the tensile strength.

# 2	Experimental procedure

## 2.1	Materials and equipment

The printing materials tested were commercial virgin and recycled PLA characterized by data listed in Table 2.1. The recycled PLA was comprised of a blend containing 10 % virgin PLA.

Table 2.1: Characterization and processing conditions of the PLA used and the recycled PLA

|  | PLA | Recycled PLA |
| --- | --- | --- |
| Composition | PLA (polylactic resin)- 99 % CAS: 9051-89-2 | PLA – 10 % CAS: 9051-89-2 and recycled PLA 90 % |
| Density | 1.24 g/cm3 | 1.1-1.3 g/cm3 |
| Diameter | 1.75 ± 0.03 mm | 1.75 mm |
| Printing temperature | 220 ± 20 ºC | 205 ± 15 ºC |
| Melting temperature | 180 ºC | 160 ± 10 ºC |

The specimens were printed with a BQ Witbox, shown in Figure 2.1a, using the Ultimaker Cura 3.2.1 software. For tensile testing, a MTS Criterion 43 universal testing machine (Figure 2.1b) was used, selecting a strain rate of 0.5 mm/min. The mechanical specimens were manufactured according to the dimensions depicted in Figure 2.1c.

(a)                                        (b)                                                           (c)

Figure 2.1: Equipment used in the study: a) 3D printer, b) Universal testing machine and c) Mechanical sample.

## 2.2	Methodology

The experimental plan included three different phases (Figure 2.2) to carry out a comprehensive study with a limited number of tests that do not compromise the reliability of the results.

The main goal of Phase I is to identify and discard factors depending on their influence on the response variable. The response variable chosen was the maximum load attained during the testing of the specimen.35,43 Fractional designs allow to minimize the number of tests, being used as screening designs. The use of random order made it possible to guarantee that the hypothesis stating that the errors are independently distributed random variables was fulfilled.44 The critical parameters for the study are the layer height (0.15 and 0.3 mm) and infill pattern (tri-hexagonal and grid).45,46 In addition, taking into account the goal of sustainable manufacturing (i.e. trying to optimize the consumption of material), but also productivity (i.e. trying to minimize printing times), infill density (60 and 100 %)25 and printing speed (40 and 80 mm/s)24,47 were considered. The printing temperature was 210 °C, which was the recommended temperature for PLA material. The design included only specimens printed in the horizontal orientation. To conclude this phase, an analysis of variance (ANOVA) allowed to identify the factors influencing the response variable.

The main goal of Phase II is to study in more detail the influence of the most influential factor according to Phase I. Threfore, the intent is to focus on how the response variable evolves by varying the most influential factors. For that reason, an extension of the factor levels was established. On the other hand, the criteria selection of levels for the other three factors aimed at minimizing the printing time.

Finally, Phase III aimed at evaluating the influence of the anisotropy based on the printing orientation, which may notably affect the tensile strength48. Because of the anisotropy, the UNE 116005:20124649 standard requires printing the specimens in three different orientations: edgewise, horizontal and vertical, testing five samples in each orientation. This phase included the printing of 15 specimens of both materials.

Figure 2.2: Summary of the three phases of the experimental plan.

# 3	Findings

## 3.1	Phase I: Screening phase

Table 3.1 summarizes the experimental strategy with the results of the maximum load attained during this screening phase. A total of 16 samples were tested.

Table 3.1: Results of the Phase I

| Material | Layer height (mm) | Infill pattern | Infill density (%) | Printing speed (mm/s) | Max load (kN) | Young´s modulus (MPa) |
| --- | --- | --- | --- | --- | --- | --- |
| Virgin | 0.15 | Tri-hex | 60 | 40 | 2.21 | 808.50 |
| Virgin | 0.3 | Tri-hex | 60 | 80 | 2.16 | 826.31 |
| Virgin | 0.15 | Grid | 60 | 80 | 2.24 | 788.51 |
| Virgin | 0.3 | Grid | 100 | 80 | 3.60 | 1,143.72 |
| Virgin | 0.3 | Tri-hex | 100 | 40 | 3.62 | 1,160.47 |
| Virgin | 0.15 | Tri-hex | 100 | 80 | 3.81 | 1,127.84 |
| Virgin | 0.15 | Grid | 100 | 40 | 3.79 | 1,132.18 |
| Virgin | 0.3 | Grid | 60 | 40 | 2.16 | 834.48 |
| Recycled | 0.15 | Tri-hex | 60 | 40 | 2.16 | 834.48 |
| Recycled | 0.3 | Tri-hex | 60 | 80 | 2.16 | 875.78 |
| Recycled | 0.3 | Grid | 60 | 40 | 2.15 | 852.18 |
| Recycled | 0.15 | Tri-hex | 100 | 80 | 3.38 | 1,058.60 |
| Recycled | 0.3 | Tri-hex | 100 | 40 | 3.37 | 1,106.42 |
| Recycled | 0.15 | Grid | 60 | 80 | 2.05 | 845.71 |
| Recycled | 0.15 | Grid | 100 | 40 | 3.53 | 1,126.03 |
| Recycled | 0.3 | Grid | 100 | 80 | 3.49 | 1,152.35 |

In general, shortly after attaining the maximum load, the fracture of the specimen occurred. However, the nature of the fracture was not homogeneous as shown in Figure 3.1a. In most cases, the specimens showed fragile behavior, and the fracture, either horizontally or with a lower inclination angle, was clean. However, for the recycled material, the specimens presented ductile behavior and, properly, the fracture did not occur after the maximum load was attained. In these cases, the tensile tests were cancelled after the maximum load was attained, without reaching a complete fracture. The breakage in these cases occurred at a 45° angle and, in the case of the RE-2 specimen, two parallel fracture lines can be clearly seen. The images of the fractured specimens did not allow to observe a clear relation of the fracture to the printing conditions. However, the fracture behavior may relate to that explained by Yao et al.24 The authors identified two different types of fracture: in-layer and interlayer. In general, the interlayer fracture occurs at the interface of two layers when printing in a vertical position, even when varying the printing orientation up to 45° from the vertical position. In-layer fracture is more likely to ocurr when using an edgewise position (or, inclined up to 45° from that position). In this case, the printing direction is the same as the tensile stress direction, which also happens when the horizontal orientation is used. In these cases, the material layer is not intact after the fracture. As a result, it is likely that both modes (in-layer and interlayer fractures) coexist in this study, which may explain the heterogeneity of the different fractures.

Table 3.2 lists the ANOVA results, obtained using R software, that allow to identify the influential factors on the maximal load and Young’s modulus. As a criterion, critical factors for the response variable were those with p-values lower than 0.05. Shapiro-Wilk normality tests were conducted to verify the normality of the residuals for both models. Thus, it can be clearly identified how only the infill density (lowest p-value) was a statistically significant factor for both the maximum load and for the Young’s modulus. Moreover, the type of material type and printing speed were also significant factors for the maximum load, though the influence of the printing is minimal as reported by Pazhamannil et al.50. The contribution to the total variance in the maximal load model was 97.3 % and 1.3 % for the infill density and type of material, respectively. In the case of the Young’ modulus, the infill presented a contribution of 96.6 %. Thus, when manufacturing new parts, infill density is a key factor for guaranteeing adequate tensile strength. The infill density was also identified as a significant factor for the tensile strength, along with the build orientation and nozzle diameter, in the study by Hikmat et al.51 The use of recycled PLA in the blend affects also the tensile strength accordingly to the study presented by Babagowda et al.36 In the study, the authors identified how the larger the percentage of recycled PLA, the lower the tensile strength. In the present study, the percentage of recycled material was 90 %, so this result provided by the ANOVA was expected. On contrary, the layer height was not found to be a statistically significant source of variation as expected22,23. Figure 3.1b illustrates the boxplots of the results considering each of the factors. In the figures, it is possible to see how the factors affect the response variables, particularly the influence of the infill density.

(a)

(b)

Figure 3: Phase I: screening tests to identify significant factors based on DoE. (a) Tensile sample of the Phase I. (b) Boxplots to identify significant factors based on DoE

Table 3.2: ANOVA results at 95 % significance level for Maximal load and Young´s modulus variables

| Variable | Maximal load |  |  |  |  | Young´s modulus |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Df | Sum Sq | Mean Sq | F value | Pr(>F) | Df | Sum Sq | Mean Sq | F value | Pr(>F) |
| Layer Height | 1 | 0.013 | 0.013 | 1.41 | 0.262 | 1 | 3302.226 | 3302.226 | 3.956 | 0.075 |
| Infill Density | 1 | 7.981 | 7.981 | 851.04 | 5.2e-11 | 1 | 342710.722 | 342710.722 | 410.611 | 1.8e-09 |
| Infill Pattern | 1 | 0.001 | 0.001 | 0.131 | 0.725 | 1 | 368.256 | 368.256 | 0.441 | 0.522 |
| Printing Speed | 1 | 0.001 | 0.001 | 0.067 | 0.007 | 1 | 80.64 | 80.64 | 0.097 | 0.762 |
| Material | 1 | 0.106 | 0.106 | 11.264 | 0.007 | 1 | 54.538 | 54.538 | 0.065 | 0.803 |
| Residuals | 10 | 0.094 | 0.009 |  |  | 10 | 8346.354 | 834.635 |  |  |

## 3.2	Phase II: Focusing

The main goal of Phase II is to evaluate in more detail the influence of the infill density on the tensile strength based on the results of Phase I. Therefore, five levels of the infill density were chosen: 40, 55, 70, 85 and 100 %. Regarding the selection of the other printing parameters, the main criterion was the reduction of the printing time. Thus, the experimental conditions were layer height of 0.3 mm, tri-hexagonal infill pattern and printing speed of 80 mm/s with an estimated printing time of 20 min. A total of 10 samples were manufactured.

Figure 4a shows the fracture of the specimens tested in Phase II. Regarding the fracture, the results were similar to those of the Phase I (i.e., more ductile behavior for the recycled PLA specimens). The interesting element in this phase is presented in Figure 4b where the maximum load versus infill density for both materials is illustrated.

(a)

(b)

Figure 4: Phase II: Evaluation of the infill density in the mechanical load. a) Mechanical samples used in Phase II. b) Adjustment of the data to two models.

From Figure 4b, the experimental data was used to create two models. In the A region, which comprises infill densities ranging from 40 to 85 %, the slope of the curve grows slowly with an approximately linear trend. In the B region, from 85 to 100 %, the increase of the tensile strength becomes more pronounced using a quadratic function. Regarding the type of material, it is clear that virgin PLA moderately outperforms recycled PLA. These results are in agreement with studies on the comparison of the performance of recycled and virgin PLA32 in which there was found to be a difference of about 10 % in the tensile strength in the first recycling cycles. However, the difference notably increased as the infill density approached 100 %. The results obtained closely match those presented by Wang et al.52 In their study, the authors studied infill densities of 20, 40, 60, 80 and 100 % and the evolution of the tensile strength is similar to that shown in Figure 4b.

Based on the results, it appears that a reduction from 100 to 40 % of the infill density implies a relatively limited reduction, on average 41.7 %, of the maximum load supported for both types of materials. Although the number of measured points is reduced, it is possible to model the relation between the maximum load and the infill density for the two tested materials by means of polynomial regressions. The models may help to anticipate the tensile strength of a part based on the infill density.

## 3.3	Phase III: Study on the printing orientation

In this final phase, the main goal is to test the influence of the building orientation according to the UNE 116005:201249,53 standard.

Five specimens for each of the orientations (edgewise, horizontal and vertical) for both materials were manufactured. The selected printing conditions were infill density of 50 %, printing speed of 80 mm/s, tri-hexagonal infill pattern and layer height of 0.3 mm, with the objective of limiting the use of material and the time required for printing. A total of 30 samples were tested.

Figure 5a shows the images of the tested specimens displaying the same type of fracture as in the first two phases. It is interesting to evaluate the reduction in the maximum load depending on the type of material and the orientation in which the specimens were printed. Thus, Figure 5b details the maximum load and the mean values for the five specimens at each orientation. From the results, it is clear that the horizontal orientation is the one that provided the higher tensile strength as found by Corapi et al.54, followed by the edgewise orientation. Likewise, the virgin samples performed better than the recycled samples. According to Kiendl and Gao55, for unidirectional layups, when the fibers are aligned with the loading direction, toughness, strength and stiffness attain the highest values.

The vertical orientation provided the worst results due to the deposition of the layers perpendicular to the tensile direction. These results correspond to those by Chacón et al.43, Corapi et al.54 and Wang et al.52 For the recycled material, there is a slight decrease in the maximum load obtained from 6.71 to 13 % depending on the orientation with respect to the virgin values. Specifically, the biggest reduction in the load takes place in the vertical orientation, which demonstrates the highest decrease at 13 %. However, the other two orientations are more adequate for substituting the virgin material for the recycled one, with a limited reduction in the tensile strength (6.71 to 7.93 %).

(a)                                                                                     (b)

Figure 5: Phase III: Evaluation of the anisotropy. a) Specimens after tensile test in Phase III. b)Average of the maximum load obtained for each build orientation.

# 4	Discussion and limits of the results

One of the systemic problems of plastic waste involves dependency of the indiscriminate disposal of plastics, which carries multiple risks because many plastic products contain additives that modify their physico-mechanical properties, making recycling/reuse difficult.56 The use of 3D printing technology for prototyping is not exempt from this societal issue. The main purpose of this article is to assess the extent to which the influence of printing parameters affects the tensile strength. While a large body of literature is focused on the optimization of the parameters for obtaining functional printed objects using 100 % of the printed material, the approach taken here is to observe the influence of a wide range of factors that are critical within conventional printing ranges. This type of approach enables designers and users to utilize printing setups that are designed for object prototypes, providing certainty about the quality of the printed products.

One of the main results of this study demonstrates that there is a reduction about 41.7% (on average) in the maximum load supported for PLA (virgin and recycled) when the infill density changes from 100 to 40 %. Moreover, it could be inferred from the results that an infill density of 40 % retained 58.1 % of the tensile strength. This is a relevant insight for prescriptions of minimal conditions for 3D printing. Moreover, the use of recycled assets in the printing process may be a relevant method, considering the current priorities of the European Union in regard to circular economy and carbon-neutral strategy ambitions.57 Also, there is great development in applications using distributed recycling approaches. For instance, Nur-A-Tomal et al.58 presented a valuable example of waste-to-wealth to use waste plastic toys retaining the original color of waste plastic to fabricate new products. Certainly more research is required for the development of complete closed-loop case studies for prototyping purposes based on the type of material, validating technical, ecological and economic feasibility.17,59

There are certain limitations to this work in the perspective of materials and parameters tested. Certainly, the use of other materials is needed to confirm the main findings. Moreover, other factors are needed in order to consider the quality of a prototype. Clearly, other variables, such as aesthetic design, dimensional accuracy and surface quality60 are also key variables to include for the printed objects in addition to the mechanical properties in the prototypes where the main goal is user acceptability.61,62 Nevertheless, this is an ongoing study in which the main purpose is the statistical validation of the minimal conditions to promote the use of recycled materials in prototyping.

# 5	Conclusions

The present study proposes a comprehensive experimental program to analyze the Fused Filament Fabrication process based on the tensile strength using virgin and recycled PLA. The paper aims to improve the sustainability of the 3D printing process, proposing a methodology based on Design of Experiments approach to assess the technical feasibility of the substitution of virgin materials for recycled ones by means of a better knowledge on the influence of the printing conditions. The final purpose in the long term is to recognize the technology affordance of prototyping side of additive manufacturing as a design tool to better ensure consumer acceptance and less waste.63

To a great extent, the printing conditions determined the tensile strength of the specimens. Specifically, the factor that most influenced the maximum load was the infill density. The influence of the infill density on the maximum load made it possible to identify two different regions: from 40 to 85 %, linear behavior with a slight slope, and from 85 to 100 % where the maximum load notably increases to a greater extent. In general, the fracture of the virgin material corresponded to that of a fragile material, while the fracture of the recycled material showed more ductile behavior.

The selected orientation for printing is of great importance because of the anisotropy. The horizontal orientation allowed to attain a higher maximum load, while the vertical orientation provided a lower value due to the fact that no layers were deposited in the tensile direction. Our results support the main argument for the substitution of virgin PLA for recycled PLA, advancing towards sustainable manufacturing. It was found that, when using an infill density of 40 %, there is a retention of 58.1 % of the tensile strength. Despite the fact that recycled PLA offers slightly lower tensile strength, by properly selecting the printing conditions, it could be close to that of the virgin PLA. Particularly, when using the edgewise and horizontal orientations. Future research needs to evaluate the quality of a (recycled) prototype including quality aspects other than tensile aspects such as aesthetics, accuracy. Moreover, the acceptability of recycled products that can be technical printable is a major milestone.

# 6	Acknowledgements

The authors would like to thank the “Mechanical and Energy Engineering” TEP 250 research group and the Lorraine Fab Living Lab. This research has received funding from the European Union’s Horizon 2020 research and innovation program under grant agreement No. 869952.

# 7	Declaration of interest statement

The authors report no declarations of interest.

# References

1. 	Singh S, Singh G, Prakash C, et al. Current status and future directions of fused filament fabrication. J Manuf Process 2020; 55: 288–306.

2. 	Sartal A, Carou D, Dorado-Vicente R, et al. Facing the challenges of the food industry: Might additive manufacturing be the answer? Proc Inst Mech Eng Part B J Eng Manuf 2019; 233: 1902–1906.

3. 	Akhoundi B, Behravesh AH, Bagheri Saed A. An innovative design approach in three-dimensional printing of continuous fiber–reinforced thermoplastic composites via fused deposition modeling process: In-melt simultaneous impregnation. Proc Inst Mech Eng Part B J Eng Manuf 2020; 234: 243–259.

4. 	Nam J, Jo N, Kim JS, et al. Development of a health monitoring and diagnosis framework for fused deposition modeling process based on a machine learning algorithm. Proc Inst Mech Eng Part B J Eng Manuf 2020; 234: 324–332.

5. 	Jiang P, Leng J, Ding K, et al. Social manufacturing as a sustainable paradigm for mass individualization. Proc Inst Mech Eng Part B J Eng Manuf 2016; 230: 1961–1968.

6. 	Askari M, Hutchins DA, Thomas PJ, et al. Additive manufacturing of metamaterials: A review. Addit Manuf 2020; 36: 101562.

7. 	Wang L, Jiang S, Zhang S. Mapping technological trajectories and exploring knowledge sources: A case study of 3D printing technologies. Technol Forecast Soc Change 2020; 161: 120251.

8. 	Niaki MK, Torabi SA, Nonino F. Why manufacturers adopt additive manufacturing technologies: The role of sustainability. J Clean Prod 2019; 222: 381–392.

9. 	Peng T, Kellens K, Tang R, et al. Sustainability of additive manufacturing: An overview on its energy demand and environmental impact. Addit Manuf 2018; 21: 694–704.

10. 	Despeisse M, Baumers M, Brown P, et al. Unlocking value for a circular economy through 3D printing: A research agenda. Technol Forecast Soc Change 2017; 115: 75–84.

11. 	González-Henríquez CM, Sarabia-Vallejos MA, Rodriguez-Hernandez J. Polymers for additive manufacturing and 4D-printing: Materials, methodologies, and biomedical applications. Prog Polym Sci 2019; 94: 57–116.

12. 	Ryberg MW, Hauschild MZ, Wang F, et al. Global environmental losses of plastics across their value chains. Resour Conserv Recycl 2019; 151: 104459.

13. 	Elverum CW, Welo T, Tronvoll S. Prototyping in New Product Development: Strategy Considerations. Procedia CIRP 2016; 50: 117–122.

14. 	Menold J, Jablokow K, Simpson T. Prototype for X (PFX): A holistic framework for structuring prototyping methods to support engineering design. Des Stud 2017; 50: 70–112.

15. 	Hansen CA, Özkil AG. From Idea to Production: A Retrospective and Longitudinal Case Study of Prototypes and Prototyping Strategies. J Mech Des; 142. Epub ahead of print March 2020. DOI: 10.1115/1.4045385.

16. 	Campbell I, Bourell D, Gibson I. Additive manufacturing: rapid prototyping comes of age. Rapid Prototyp J 2012; 18: 255–258.

17. 	Cruz Sanchez FA, Boudaoud H, Camargo M, et al. Plastic recycling in additive manufacturing: A systematic literature review and opportunities for the circular economy. J Clean Prod 2020; 264: 121602.

18. 	Mikula K, Skrzypczak D, Izydorczyk G, et al. 3D printing filament as a second life of waste plastics—a review. Environ Sci Pollut Res. Epub ahead of print September 2020. DOI: 10.1007/s11356-020-10657-8.

19. 	Lovo JFP, Fortulan CA, Silva MM da. Optimal deposition orientation in fused deposition modeling for maximizing the strength of three-dimensional printed truss-like structures. Proc Inst Mech Eng Part B J Eng Manuf 2019; 233: 1206–1215.

20. 	Laureto JJ, Pearce JM. Anisotropic mechanical property variance between ASTM D638-14 type i and type iv fused filament fabricated specimens. Polym Test 2018; 68: 294–301.

21. 	Popescu D, Zapciu A, Amza C, et al. FDM process parameters influence over the mechanical properties of polymer specimens: A review. Polym Test 2018; 69: 157–166.

22. 	Tymrak BM, Kreiger M, Pearce JM. Mechanical properties of components fabricated with open-source 3-D printers under realistic environmental conditions. Mater Des 2014; 58: 242–246.

23. 	Altan M, Eryildiz M, Gumus B, et al. Effects of process parameters on the quality of PLA products fabricated by fused deposition modeling (FDM): Surface roughness and tensile strength. Mater Test 2018; 60: 471–477.

24. 	Yao T, Deng Z, Zhang K, et al. A method to predict the ultimate tensile strength of 3D printing polylactic acid (PLA) materials with different printing orientations. Compos Part B Eng 2019; 163: 393–402.

25. 	Alafaghani A aldin, Qattawi A. Investigating the effect of fused deposition modeling processing parameters using Taguchi design of experiment method. J Manuf Process 2018; 36: 164–174.

26. 	Little HA, Tanikella NG, J. Reich M, et al. Towards Distributed Recycling with Additive Manufacturing of PET Flake Feedstocks. Materials (Basel) 2020; 13: 4273.

27. 	Zhao P, Rao C, Gu F, et al. Close-looped recycling of polylactic acid used in 3D printing: An experimental investigation and life cycle assessment. J Clean Prod 2018; 197: 1046–1055.

28. 	Petrovic V, Vicente Haro Gonzalez J, Jordá Ferrando O, et al. Additive layered manufacturing: sectors of industrial application shown through case studies. Int J Prod Res 2011; 49: 1061–1079.

29. 	Wittbrodt BT, Glover AG, Laureto J, et al. Life-cycle economic analysis of distributed manufacturing with open-source 3-D printers. Mechatronics 2013; 23: 713–726.

30. 	Santander P, Cruz Sanchez FA, Boudaoud H, et al. Closed loop supply chain network for local and distributed plastic recycling for 3D printing: a MILP-based optimization approach. Resour Conserv Recycl 2020; 154: 104531.

31. 	Suárez L, Domínguez M. Sustainability and environmental impact of fused deposition modelling (FDM) technologies. Int J Adv Manuf Technol 2020; 106: 1267–1279.

32. 	Cruz Sanchez FA, Boudaoud H, Hoppe S, et al. Polymer recycling in an open-source additive manufacturing context: Mechanical issues. Addit Manuf 2017; 17: 87–105.

33. 	Lanzotti A, Martorelli M, Maietta S, et al. A comparison between mechanical properties of specimens 3D printed with virgin and recycled PLA. Procedia CIRP 2019; 79: 143–146.

34. 	Anderson I. Mechanical Properties of Specimens 3D Printed with Virgin and Recycled Polylactic Acid. 3D Print Addit Manuf 2017; 4: 110–115.

35. 	Kumar R, Singh R, Farina I. On the 3D printing of recycled ABS, PLA and HIPS thermoplastics for structural applications. PSU Res Rev 2018; 2: 115–137.

36. 	C, Kadadevara Math RS, Goutham R, et al. Study of Effects on Mechanical Properties of PLA Filament which is blended with Recycled PLA Materials. IOP Conf Ser Mater Sci Eng 2018; 310: 012103.

37. 	Pinho AC, Amaro AM, Piedade AP. 3D printing goes greener: Study of the properties of post-consumer recycled polymers for the manufacturing of engineering components. Waste Manag 2020; 118: 426–434.

38. 	Zhao XG, Hwang K-J, Lee D, et al. Enhanced mechanical properties of self-polymerized polydopamine-coated recycled PLA filament used in 3D printing. Appl Surf Sci 2018; 441: 381–387.

39. 	Rebaioli L, Fassi I. A review on benchmark artifacts for evaluating the geometrical performance of additive manufacturing processes. Int J Adv Manuf Technol 2017; 93: 2571–2598.

40. 	Cruz Sanchez FA, Boudaoud H, Muller L, et al. Towards a standard experimental protocol for open source additive manufacturing. Virtual Phys Prototyp 2014; 9: 151–167.

41. 	Roberson Da, Espalin D, Wicker RB. 3D printer selection: A decision-making evaluation and ranking model. Virtual Phys Prototyp 2013; 8: 201–212.

42. 	Jaisingh Sheoran A, Kumar H. Fused Deposition modeling process parameters optimization and effect on mechanical properties and part quality: Review and reflection on present research. In: Mater. Today proc. 2020, pp. 1659–1672.

43. 	Chacón JM, Caminero MA, García-Plaza E, et al. Additive manufacturing of PLA structures using fused deposition modelling: Effect of process parameters on mechanical properties and their optimal selection. Mater Des 2017; 124: 143–157.

44. 	Montgomery DC. Design and Analysis of Experiments. John Wiley; Sons Inc, 2001.

45. 	Singh R, Singh H, Farina I, et al. On the additive manufacturing of an energy storage device from recycled material. Compos Part B Eng 2019; 156: 259–265.

46. 	Tanveer MdQ, Haleem A, Suhaib M. Effect of variable infill density on mechanical behaviour of 3-D printed PLA specimen: an experimental investigation. SN Appl Sci 2019; 1: 1–12.

47. 	Pérez M, Medina-Sánchez G, García-Collado A, Gupta M, Carou D. Surface Quality Enhancement of Fused Deposition Modeling (FDM) Printed Samples Based on the Selection of Critical Printing Parameters. Materials 2018, 11 :1382.

48. Zhao Y, Chen Y, Zhou Y, Novel mechanical models of tensile strength and elastic property of FDM AM PLA materials: Experimental and theoretical analyses. Mater Des 2019, 181 :108089

49. UNE. UNE 116005:2012 Fabricación por adición de capas en materiales plásticos. Fabricación aditiva. Preparación de probetas.

50. 	Pazhamannil RV, Govindan P, Sooraj P. Prediction of the tensile strength of polylactic acid fused deposition models using artificial neural network technique. Materials Today: Proceedings 2021, 46(19) :9187-9193,

51. Hikmat M, Rostam S, Ahmed YM. Investigation of tensile property-based Taguchi method of PLA parts fabricated by FDM 3D printing technology. Results Eng 2021 ; 11 :100264

52. Wang S, Ma Y, Deng Z, et al. Effects of fused deposition modeling process parameters on tensile, dynamic mechanical properties of 3D printed polylactic acid materials. Polym Test 2020; 86: 106483.

53. 	García-Domínguez A, Claver J, Camacho AM, et al. Considerations on the Applicability of Test Methods for Mechanical Characterization of Materials Manufactured by FDM. Materials 2019; 13: 28.

54. 	Corapi D, Morettini G, Pascoletti G, et al. Characterization of a polylactic acid (PLA) produced by Fused Deposition Modeling (FDM) technology. In: Procedia struct. integr. Elsevier B.V., 2019, pp. 289–295.

55. 	Josef Kiendl, Chao Gao, Controlling toughness and strength of FDM 3D-printed PLA components through the raster layup. Composites Part B: Engineering 2020, 180:107562

56. Wagner S, Schlummer M. Legacy additives in a circular economy of plastics: Current dilemma, policy analysis, and emerging countermeasures. Resour Conserv Recycl 2020; 158: 104800.

57. 	Schwarz AE, Ligthart TN, Godoi Bizarro D, et al. Plastic recycling in a circular economy; determining environmental performance through an LCA matrix model approach. Waste Manag 2021; 121: 331–342.

58. 	Nur-A-Tomal MS, Pahlevani F, Sahajwalla V. Direct transformation of waste children’s toys to high quality products using 3D printing: A waste-to-wealth and sustainable approach. J Clean Prod; 267. Epub ahead of print 2020. DOI: 10.1016/j.jclepro.2020.122188.

59. 	Sauerwein M, Doubrovski E, Balkenende R, et al. Exploring the potential of additive manufacturing for product design in a circular economy. J Clean Prod 2019; 226: 1138–1149.

60. 	Jin Y, Wan Y, Zhang B, et al. Modeling of the chemical finishing process for polylactic acid parts in fused deposition modeling and investigation of its tensile properties. J Mater Process Technol 2017; 240: 233–239.

61. 	Sauer J, Sonderegger A. The influence of prototype fidelity and aesthetics of design in usability tests: Effects on user behaviour, subjective evaluation and emotion. Appl Ergon 2009; 40: 670–677.

62. 	Sauer J, Seibel K, Rüttinger B. The influence of user expertise and prototype fidelity in usability tests. Appl Ergon 2010; 41: 130–140.

63. 	Kohtala C. Addressing sustainability in research on distributed production: an integrated literature review. J Clean Prod 2015; 106: 654–668.

University of Jaén, Department of Mechanical and Mining Engineering, 23071 Jaén, Spain

Universidade de Vigo, Departamento de Deseño na Enxeñaría, Ourense, Spain, diecapor@uvigo.es

Université de Lorraine - ERPI - F-54000, Nancy, France, cruzsanc1@univ-lorraine.fr